# Resilient Volusia FDEP CPI

# Coastal Flooding/ Surge Modeling Methods

# University of Florida GeoPlan Center

Sea level rise (SLR) is expected to exacerbate coastal flooding events, increasing the extent and depth of flooding as well as the intensity of damage from storms.  For this project, researchers from the University of Florida GeoPlan Center looked at two methods for modeling future coastal flood risk under sea level rise scenarios using FEMA’s Hazus-MH software.  The goal of this modeling was to estimate the depth and extent of coastal flooding under future SLR conditions and assess those impacts to the transportation network.  This document outlines the methods tested for the Volusia County pilot area, as part of the Resilient Volusia FDEP Coastal Partnership Initiative grant.

## Background: Hazus-MH - Multi-Hazard Loss Estimation Methodology

The Multi-Hazard Loss Estimation Methodology (Hazus-MH) is a standardized methodology created by the Federal Emergency Management Administration (FEMA) to estimate potential losses from earthquakes, hurricane winds, and floods.  Hazus-MH contains multiple models for estimating losses from various hazards, including the Earthquake Model, the Flood Model, and the Hurricane Model.

The Flood Model is used to estimate riverine and coastal flood hazards and potential damage to buildings, infrastructure, and land use.   The software models specific return intervals of flooding (such as the 100-year return interval or commonly referred to “100-year flood event” or “100-year storm”) and damages resulting from those events. The Hurricane Wind Model is used to estimate hurricane winds and potential damage to buildings for the Atlantic and Gulf Coast regions and Hawaii.  There is also an option to model the combined flood and wind hazards to produce hurricane-induced coastal surge.  FEMA’s Hazus-MH website: https://www.fema.gov/hazus

## Method 1: Hazus-MH Coastal Flood Hazards Model:

## Analyzing 100-year storms-surge with SLR Scenarios

The first method to model potential future storm surge utilized the Coastal Flood Hazards component of the Hazus-MH Flood Model. This model follows FEMA’s methodology for developing the Flood Insurance Rate Maps (FIRMs), where flood frequency and flood magnitude (or depth) are used to define flood hazard. The model relies on the 100-year Stillwater elevation (SWEL) and Stillwater depth (SWD) to identify the inland impacts of storm surge.  Coastal flooding scenarios were run on the entire Atlantic coastline of Volusia County using the FEMA 100-year or 1% annual chance flood event as a baseline scenario.  Nine scenarios of future SLR were then modeled by adding the SLR amounts to the coastal Stillwater elevations for the base 100-year storm.

The SLR scenarios used for these model runs are from the U.S. Army Corps of Engineers, and are consistent with the modeling and vulnerability analysis conducted by the East Central Florida Regional Planning Council for other counties and communities in the east central Florida region. In Table 1 (below), the SLR values are listed in feet, relative to mean sea level (MSL), and utilizing local sea level trend information from the Daytona Beach Shores tide station. At 2040, the range of projected SLR is from 0.37-feet to 1.22-feet. At 2070, the range of projected SLR is 0.59-feet to 2.85-feet. At 2100, the range of projected SLR is from 0.82-feet to 5.15-feet.  Figure 1 below is a graph plotting the SLR values in Table 1.

| USACE SLR Projections Feet of RSLR (relative to MSL) Daytona Beach Shores, FL |  |  |  |
| --- | --- | --- | --- |
|  | Low | Intermediate | High |
| 2040 | 0.37 ft | 0.57 ft | 1.22 ft |
| 2070 | 0.59 ft | 1.14 ft | 2.85 ft |
| 2100 | 0.82 ft | 1.86 ft | 5.15 ft |

Table 1. U.S. Army Corps of Engineers Sea Level Rise Projections in Feet, Daytona Beach Shores, FL

Figure 1.  Relative Sea Level Change Projections, Daytona Beach Shores, FL, U.S. Army Corps of Engineers sea-level change calculator

The Hazus-MH software allows the user to enter the Stillwater elevations for different flooding event return rates (for example: 10% annual chance, 2% annual chance, 1% annual chance, 0.2% annual chance).  SWELs for the 1% annual chance flood were obtained from the 2014 FEMA Flood Insurance Study (FIS) for Volusia County.  For Volusia County 2014 FIS, there are 49 coastal transects; each lists the 1% annual chance SWEL of 6.9 Feet NAVD88.

For each of the nine scenarios, the relative SLR amount was added to the Stillwater Elevation (SWEL), to represent a future SWEL. The Coastal Flood Hazard Model was then run with the future SWEL.  A 5-meter Digital Elevation Model (DEM), compiled with Lidar data by the GeoPlan Center, was used for the modeling.  The model outputs include depth grids of the 100-year storm surge, which were then overlaid with roads data to analyze impacts to the road network under each of the nine scenarios.   The roads analysis does not include the surge depth on each road segment, but rather a summary of amount and proportion of the roadway affected under each future scenario.

## Method 2: HAZUS-MH Coastal Surge Model:

## Modeling Historic Hurricanes plus SLR

The second method to model future storm surge also utilized FEMA’s Hazus-MH, but a different model – the Coastal Surge Model. This model is a more recent addition to Hazus and couples storm surge and wave modeling functionality, using SLOSH for storm surge and SWAN for wave heights.

The purpose of the model is to develop an overall estimate of combined coastal wind and flood losses for a single hurricane event.  Previous Hazus-MH functionality allowed for wind-only or flood-only loss analysis.

The UF GeoPlan Center investigated this method as a way to model historic storms with the addition of SLR, to evaluate how SLR might change the extent and depth of flooding from a single storm event.  For the Volusia County study area, a historic storm was selected from the available historical hurricanes in Hazus-MH Coastal Surge Model.  One major limitation of the model is that only historical hurricanes of a Category 3 and higher are available, which greatly reduced the selection pool of hurricanes.  Historically, there has been a low frequency of Category 3 or higher hurricanes making landfall along the Volusia County.  The closest historic hurricane available in the Hazus-MH hurricane model was Hurricane Dora, which made landfall near St. Augustine in 1964.  The expected storm surge from this historic hurricane was low, because of the landfall location (considerably north of Volusia County) and the surge was not from the right front quadrant, where the greatest surge is typically expected.

First, the coastal surge model was run for Hurricane Dora to represent a baseline representation of historical storm surge. Next, increments of SLR were added to the initial water level when running the hurricane model to simulate the same hurricane making landfall from higher sea levels.

Maps of storm surge from the resulting model runs are available for viewing in the beta map viewer.  It is the project team’s recommendation that the Coastal Surge Model needs more investigation and that another geographic area, which has historically experienced more direct hits from hurricanes, be modeled.